VOCABULARY

wondering 			удивление

wave energy 		энергия волн

energy converter 		преобразователь энергии

electrical power 		электрическая энергия

seabed ['siːbed] 		морское дно

flexible disc joint  		подвижное дисковое соединеие

lever                   		 рычаг, средство воздействия

chamber                            хранилище

a set of pistons                  комплект поршней

water pipes                        трубы

turbine                               турбина

pounds per square inch     фунтов на квадратный дюйм

oscillate ['ɔsɪleɪt]                качать

linear ['lɪnɪə]                       линейный

reciprocate [rɪ'sɪprəkeɪt]      двигаться взад и вперёд; совершать возвратно-

поступательные движения

rotate [rəu'teɪt]		     вращаться

account of the operation of the system   описание работы системы

renewable [rɪ'njuːəbl] 	восстановимый, возобновляемый (о природных ресурсах и т. п.)

SAMPLE OF PRESENTATION

Good morning, everyone. Today I am going to talk about a Wave Energy Converter.

You are probably wondering what a Wave Energy Converter is. So, let us have a simple definition to start with. Very simply, a Wave Energy Converter is a system, which converts the energy from sea waves into electrical power.

Before I talk about the system itself, let me tell you where it is located, because some systems are located on the surface of the sea, and some on the seashore. But not this system. The Wave Energy Converter is fixed to the seabed.

OK, now let us look at the main components, The Wave Energy Converter has five main components or parts. These are: a very large flexible disc, a lever, a chamber which takes in sea water, a set of pistons, many sea water pipes, and of course a turbine on the land.

The main specifications of the system are as follows. The whole system on the seabed is 4.6 metres high and 20.4 metres long; the main pipe is 125 millimetres wide; the pressure of the water in the pipes is 7000 kilopascals, or 1000 psi, that is pounds per square inch. The complete system can generate 100 kilowatts of electricity.

OK that is enough number-crunching. Let us look at how the system works. Here is a very simple account of the operation of the system. Let us start with the sea. The sea wave oscillates. This oscillating motion pushes the disc down in a linear motion. The disc makes the lever oscillate. The oscillating lever makes the pistons move in a reciprocating motion. Then the pistons push seawater from the chamber through the pipe at high pressure. The high-pressure water then makes the turbine rotate. And of course this generates electricity.

So, that is how it works. And of course, this system has great benefits. The most important benefit is that wave energy is a renewable energy resource; and of course, it uses no fossil fuels.