1 Automation

Automation is the application of machines to tasks once performed by human beings or, increasingly, to tasks that would otherwise be impossible. Although the term mechanization is often used to refer to the simple replacement of human labor by machines, automation generally implies the integration of machines into a self-governing system. Automation has revolutionized those areas in which it has been introduced, and there is scarcely an aspect of modern life that has been unaffected by it.

The term automation was coined in the automobile industry about 1946 to describe the increased use of automatic devices and controls in mechanized production lines. The origin of the word is attributed to D.S. Harder, an engineering manager at the Ford Motor Company at the time. The term is used widely in a manufacturing context, but it is also applied outside manufacturing in connection with a variety of systems in which there is a significant substitution of mechanical, electrical, or computerized action for human effort and intelligence.

In general usage, automation can be defined as a technology concerned with performing a process by means of programmed commands combined with automatic feedback control to ensure proper execution of the instructions. The resulting system is capable of operating without human intervention. The development of this technology has become increasingly dependent on the use of computers and computer-related technologies. Consequently, automated systems have become increasingly sophisticated and complex. Advanced systems represent a level of capability and performance that surpass in many ways the abilities of humans to accomplish the same activities.

Automation technology has matured to a point where a number of other technologies have developed from it and have achieved a recognition and status of their own. Robotics is one of these technologies; it is a specialized branch of automation in which the automated machine possesses certain anthropomorphic, or humanlike, characteristics. The most typical humanlike characteristic of a modern industrial robot is its powered mechanical arm. The robot's arm can be programmed to move through a sequence of motions to perform useful tasks, such as loading and unloading parts at a production machine or making a sequence of spot-welds on the sheet-metal parts of an automobile body during assembly. As these examples suggest, industrial robots are typically used to replace human workers in factory operations.

- Match the following words and word combinations with their definitions:

| - Perform |  |
| --- | --- |
| - replacement |  |
| - Assembly |  |
| - Attributed |  |
| - sophisticated |  |
| - Accomplish |  |
| - spot-welds |  |

- Say whether the following statements are true or false:

- Automation is the application of machines to tasks once performed by human beings or, increasingly, to tasks that would otherwise be impossible. A feedback control system consists of five basic components.

- The term is not used widely in a manufacturing context, but it is also applied outside manufacturing in connection with a variety of systems in which there is a significant substitution of mechanical, electrical, or computerized action for human effort and intelligence.

- Robotics is only one technology; which is not specialized branch of automation in which the automated machine possesses certain    anthropomorphic, or humanlike, characteristics.

- Match heads and tales:

| - Although the term mechanization is often used to refer to the simple replacement of human labor by machines, … | - … performance that surpass in many ways the abilities of humans to accomplish the same activities. |
| --- | --- |
| - The origin of the word is attributed to D.S. Harder, … | - … automation generally implies the integration of machines into a self-governing system. |
| - Advanced systems represent a level of capability and | - … industrial robots are typically used to replace human workers in factory operations. |
| - … Robotics is one of these technologies; | - .... it is a specialized branch of automation in which the automated machine possesses certain anthropomorphic, or humanlike, characteristics. |
| - The robot's arm can be programmed to move through a sequence of motions to perform useful tasks, | - …. an engineering manager at the Ford Motor Company at the time.. |
| - . As these examples suggest, | - …. such as loading and unloading parts at a production machine or making a sequence of spot-welds on the sheet-metal parts of an automobile body during assembly. |

- Answer the questions:

- What is the automation?

- What is the origin of this word?

- What do you know about robotics?

- Compose the sentence from the given words.

1. Those in automation in has revolutionized areas which it has been introduced.

2. These technologies robotics is one of .

3. The resulting system capable of operating human is intervention without.

- Make a short summary of the text.

###### 2 Historical development of automation

The technology of automation has evolved from the related field of mechanization, which had its beginnings in the Industrial Revolution. Mechanization refers to the replacement of human (or animal) power with mechanical power of some form. The driving force behind mechanization has been humankind's propensity to create tools and mechanical devices.

The first tools made of stone represented prehistoric man's attempts to direct his own physical strength under the control of human intelligence. Thousands of years were undoubtedly required for the development of simple mechanical devices and machines such as the wheel, the lever, and the pulley, by which the power of human muscle could be magnified. The next extension was the development of powered machines that did not require human strength to operate. Examples of these machines include waterwheels, windmills, and simple steam-driven devices. More than 2,000 years ago the Chinese developed trip-hammers powered by flowing water and waterwheels. The early Greeks experimented with simple reaction motors powered by steam. The mechanical clock, representing a rather complex assembly with its own built-in power source (a weight), was developed about 1335 in Europe. Windmills, with mechanisms for automatically turning the sails, were developed during the Middle Ages in Europe and the Middle East.

The earliest steam engines required a person to open and close the valves, first to admit steam into the piston chamber and then to exhaust it. Later a slide valve mechanism was devised to automatically accomplish these functions. The only need of the human operator was then to regulate the amount of steam that controlled the engine's speed and power. This requirement for human attention in the operation of the steam engine was eliminated by the flying-ball governor. Invented by James Watt in England, this device consisted of a weighted ball on a hinged arm, mechanically coupled to the output shaft of the engine. As the rotational speed of the shaft increased, centrifugal force caused the weighted ball to be moved outward. This motion controlled a valve that reduced the steam being fed to the engine, thus slowing the engine. The flying-ball governor remains an elegant early example of a negative feedback control system, in which the increasing output of the system is used to decrease the activity of the system.

Negative feedback is widely used as a means of automatic control to achieve a constant operating level for a system. A common example of a feedback control system is the thermostat used in modern buildings to control room temperature. In this device, a decrease in room temperature causes an electrical switch to close, thus turning on the heating unit. As room temperature rises, the switch opens and the heat supply is turned off. The thermostat can be set to turn on the heating unit at any particular set point.

Another important development in the history of automation was the Jacquard loom, which demonstrated the concept of a programmable machine. About 1801 the French inventor Joseph-Marie Jacquard devised an automatic loom capable of producing complex patterns in textiles by controlling the motions of many shuttles of different coloured threads.

The concept of programming a machine was further developed later in the 19th century when Charles Babbage, an English mathematician, proposed a complex, mechanical “analytical engine” that could perform arithmetic and data processing. Although Babbage was never able to complete it, this device was the precursor of the modern digital computer.

- Match the following words and word combinations with their definitions:

| 1. refer | a) |
| --- | --- |
| 2. the driving force | b) |
| 3. lever | c) |
| 4. wheel | d) |
| 5. waterwheels | e) |
| 6. trip-hammers | f) |
| 7. rotational speed | g) |
| 8. coloured threads | h) |

- Say whether the following statements are true or false:

- The technology of automation has evolved from the related field of mechanization, which had its beginnings in the Agricultural Revolution..

- Thousands of years were undoubtedly required for the development of simple mechanical devices and machines such as the wheel, the lever, and the pulley, by which the power of human muscle could be magnified.

- More than 5,000 years ago the Chinese developed trip-hammers powered by flowing water and waterwheels.

- Match heads and tales:

| - The technology of automation has evolved from the related field of mechanization… | - …the development of powered machines that did not require human strength to operate |
| --- | --- |
| - The next extension was | - … automatic control to achieve a constant operating level for a system. |
| - Negative feedback is widely used as a means of | - … which had its beginnings in the Industrial Revolution. |
| - The earliest steam engines required a person to open and close the valves, … | - ... Jacquard loom, which demonstrated the concept of a programmable machine... |
| - Another important development in the history of automation was the | - …. this device was the precursor of the modern digital computer. |
| - Although Babbage was never able to complete it, | - first to admit steam into the piston chamber and then to exhaust it ….. |

- Answer the questions:

- Where does mechanization refer to?

- Can you tell about the first tools?

- What did the early Greeks experiment?

- What do you know about negative feedback ?

- Compose the sentence from the given words.

1. The driving humankind's behind mechanization has been force propensity to create tools and mechanical devices.

2. Negative feedback widely used as a is control to achieve a constant operating level for means of automatic a system..

3. Although this device Babbage never able to complete it, was the precursor of the modern digital was computer.

- Make a short summary of the text.

###### 3 Modern developments

A number of significant developments in various fields have occurred during the 20th century: the digital computer, improvements in data-storage technology and software to write computer programs, advances in sensor technology, and the derivation of a mathematical control theory. All these developments have contributed to progress in automation technology.

The development of integrated circuits in the 1960s propelled a trend toward miniaturization in computer technology that has led to machines that are much smaller and less expensive than their predecessors yet are capable of performing calculations at much greater speeds. This trend is represented today by the microprocessor, a miniature multicircuited device capable of performing all the logic and arithmetic functions of a large digital computer.

Along with the advances in computer technology, there have been parallel improvements in program storage technology for containing the programming commands. Modern storage media include magnetic tapes and disks, magnetic bubble memories, optical data storage read by lasers, videodisks, and electron beam-addressable memory systems. In addition, improvements have been made in the methods of programming computers (and other programmable machines). Modern programming languages are easier to use and are more powerful in their data-processing and logic capabilities.

Advances in sensor technology have provided a vast array of measuring devices that can be used as components in automatic feedback control systems. These devices include highly sensitive electromechanical probes, scanning laser beams, electrical field techniques, and machine vision. Some of these sensor systems require computer technology for their implementation. Machine vision, for example, requires the processing of enormous amounts of data that can be accomplished only by high-speed digital computers. This technology is proving to be a versatile sensory capability for various industrial tasks, such as part identification, quality inspection, and robot guidance.

Finally, there has evolved since World War II a highly advanced mathematical theory of control systems. The theory includes traditional negative feedback control, optimal control, adaptive control, and artificial intelligence. Traditional feedback control theory makes use of linear ordinary differential equations to analyze problems, as in Watt's flying-ball governor. Although most processes are more complex than the flying-ball governor, they still obey the same laws of physics that are described by differential equations. Optimal control theory and adaptive control theory are concerned with the problem of defining an appropriate index of performance for the process of interest and then operating it in such a manner as to optimize its performance. The difference between optimal and adaptive control is that the latter must be implemented under conditions of a continuously changing and unpredictable environment; it therefore requires sensor measurements of the environment to implement the control strategy.

Artificial intelligence is an advanced field of computer science in which the computer is programmed to exhibit characteristics commonly associated with human intelligence. These characteristics include the capacity for learning, understanding language, reasoning, solving problems, rendering expert diagnoses, and similar mental capabilities. Developments in artificial intelligence are expected to provide robots and other “intelligent” machines with the ability to communicate with humans and to accept very high-level instructions rather than the detailed step-by-step programming statements typically required of today's programmable machines. For example, a robot of the future endowed with artificial intelligence might be capable of accepting and executing the command “assemble the product.” Present-day industrial robots must be provided with a detailed set of instructions specifying the locations of the product's components, the order in which they are to be assembled, and so forth.

- Match the following words and word combinations with their definitions:

| - improvements |  |
| --- | --- |
| - software |  |
| - lasers |  |
| - rendering |  |
| - locations |  |
| - unpredictable |  |
| - measurements |  |

- Say whether the following statements are true or false:

- A number of significant developments in various fields have occurred during the 20th century: the digital For example, a robot of the future endowed with artificial intelligence might be capable of accepting and executing the command “assemble the product.”  improvements in data-storage technology and software to write computer programs, advances in sensor technology, and the derivation of a mathematical control theory.

- Advances in sensor technology have provided a vast array of measuring devices that cannot be used as components in automatic feedback control systems.

- For example, a adaptive control of the future endowed with artificial intelligence might be capable of accepting and executing the command “assemble the product.”

- Match heads and tales:

| - All these developments have | - can be used as components in automatic feedback control systems. These devices include highly sensitive electromechanical probes, scanning laser beams, electrical field techniques, and machine vision. |
| --- | --- |
| - This trend is represented today | - the computer is programmed to exhibit characteristics commonly associated with human intelligence. |
| - Advances in sensor technology have provided a vast array of measuring devices that | - World War II a highly advanced mathematical theory of control systems. |
| - Finally, there has evolved since | - capable of accepting and executing the command “assemble the product.” |
| - Artificial intelligence is an advanced field of computer science in which | - by the microprocessor, a miniature multicircuited device capable of performing all the logic and arithmetic functions of a large digital computer. |
| - For example, a robot of the future endowed with artificial intelligence might be | - contributed to progress in automation technology |

- Answer the questions:

- What kind of developments have contributed to progress in automation technology?

- Must be present-day industrial robots provided with a detailed set of instructions?

- What do you know about control systems?

- Compose the sentence from the given words.

1. Along computer technology with the advances in, there have been in program storage technology for containing parallel improvements the programming commands.

2. These include characteristics the capacity for, understanding language, reasoning, solving problems, rendering, and similar mental expert diagnoses capabilities learning.

3. Present-day industrial robots must provided be with a of instructions specifying the detailed set locations order of the product's components, the in which they are to be assembled, and so forth.

- Make a short summary of the text.

###### 4 Principles and theory of automation

There are three basic building blocks of automation: (1) a source of power to perform some action, (2) feedback controls, and (3) machine programming. Almost without exception, an automated system will exhibit all these elements.

###### Power source. The actions performed by automated systems are generally of two types: (1) processing and (2) transfer and positioning. In the first case, energy is applied to accomplish some processing operation on some entity. The process may involve the shaping of metal, the molding of plastic, the switching of electrical signals in a communication system, or the processing of data in a computerized information system. All these actions entail the use of energy to transform the entity (e.g., the metal, plastic, electrical signals, or data) from one state or condition into another more valuable state or condition. The second type of action—transfer and positioning—is most readily seen in automated manufacturing systems designed to perform work on a product. In these cases, the product must generally be moved (transferred) from one location to another during the series of processing steps.

###### Feedback controls are widely used in modern automated systems. A feedback control system consists of five basic components: (1) input, (2) process being controlled, (3) output, (4) sensing elements, and (5) controller and actuating devices. These five components are illustrated in Figure 1.

The input to the system is the reference value (опорное значение), or set point (заданное значение), for the system output (выход системы). This represents the desired operating value  (параметр) of the output. Using the previous example of the heating system as an illustration, the input is the desired temperature setting for a room. The process being controlled is the heater (e.g., furnace). In other feedback systems, the process might be a manufacturing operation, the rocket engines on a space shuttle, the automobile engine in cruise control, or any of a variety of other processes to which power is applied. The output is the variable of the process that is being measured and compared to the input; in the above example, it is room temperature.

The sensing elements (датчик) are the measuring devices used in the feedback loop to monitor the value of the output variable. In the heating system example, this function is normally accomplished using a bimetallic strip. This device consists of two metal strips joined along their lengths. The two metals possess different thermal expansion coefficients; thus, when the temperature of the strip is raised, it flexes in direct proportion to the temperature change. As such, the bimetallic strip is capable of measuring temperature.

The purpose of the controller and actuating devices (исполнительное устройство) in the feedback system is to compare the measured output value with the reference input value (величина контрольного ввода) and to reduce the difference between them. In general, the controller and actuator of the system are the mechanisms by which changes in the process are accomplished to influence the output variable. These mechanisms are usually designed specifically for the system and consist of devices such as motors, valves, solenoid switches, piston cylinders, gears, power screws (ходовой винт), pulley systems, chain drives (цепная передача), and other mechanical and electrical components.

Machine programming. The programmed instructions determine the set of actions that is to be accomplished automatically by the system. The program specifies what the automated system should do and how its various components must function in order to accomplish the desired result. The content of the program varies considerably from one system to the next. In relatively simple systems, the program consists of a limited number of well-defined actions that are performed continuously and repeatedly in the proper sequence with no deviation from one cycle to the next. In more complex systems, the number of commands could be quite large, and the level of detail in each command could be significantly greater. In relatively sophisticated systems, the program provides for the sequence of actions to be altered in response to variations in raw materials or other operating conditions.

Programming commands are related to feedback control in an automated system in that the program establishes the sequence of values for the inputs (set points) of the various feedback control loops that make up the automated system. A given programming command may specify the set point for the feedback loop, which in turn controls some action that the system is to accomplish. In effect, the purpose of the feedback loop is to verify that the programmed step has been carried out. For example, in a robot controller, the program might specify that the arm is to move to a designated position, and the feedback control system is used to verify that the move has been correctly made. The relationship of program control and feedback control in an automated system is illustrated in Figure 2.

- Match the following words and word combinations with their definitions:

| - molding | - a person or thing that directs or regulates something |
| --- | --- |
| - accomplish | - a machine capable of carrying out a complex series of actions automatically, especially one programmable by a computer |
| - valve | - achieve or complete successfully |
| - controller | - material such as wood, plastic, or stone shaped for use as a decorative or architectural feature |
| - loop | - direct materials used in a production process, which are at a low level of completion compared to the final product or cost unit . |
| - robot | - a shape produced by a curve that bends round and crosses itself |
| - raw materials | - a device for controlling the passage of fluid through a pipe or duct, especially an automatic device allowing movement in one direction only |

- Say whether the following statements are true or false:

- There are thirty basic building blocks of automation

- A feedback control system consists of five basic components.

- The programmed instructions determine the set of actions that is to be accomplished automatically by the system.

- Match heads and tales:

| - Almost without exception, an automated system … | - …from one system to the next |
| --- | --- |
| - In the heating system example, this function… | - …will exhibit all these elements |
| - The content of the program varies considerably… | - … is to compare the measured output value with the reference input value and to reduce the difference between them. |
| - The program specifies what the automated system should do… | - ...is normally accomplished using a bimetallic strip. |
| - The purpose of the controller and actuating devices in the feedback system… | - …. joined along their lengths. |
| - This device consists of two metal strips… | - …and how its various components must function. |

- Think for a while and be ready to speak on the following topics:

- Power source as one of the basic building blocks of automation?

- Feedback controls as one of the basic building blocks of automation?

- Machine programming as one of the basic building blocks of automation?

- Make a short summary of the text.

###### 5 Manufacturing applications of automation and robotics

One of the most important application areas for automation technology is manufacturing. To many people, automation means manufacturing automation. Three types of automation in production can be distinguished: (1) fixed automation, (2) programmable automation, and (3) flexible automation.

Fixed automation, also known as “hard automation,” refers to an automated production facility in which the sequence of processing operations is fixed by the equipment configuration. In effect, the programmed commands are contained in the machines in the form of cams, gears, wiring, and other hardware that is not easily changed over from one product style to another. This form of automation is characterized by high initial investment and high production rates. It is therefore suitable for products that are made in large volumes. Examples of fixed automation include machining transfer lines found in the automotive industry, automatic assembly machines, and certain chemical processes.

Programmable automation is a form of automation for producing products in batches. The products are made in batch quantities ranging from several dozen to several thousand units at a time. For each new batch, the production equipment must be reprogrammed and changed over to accommodate the new product style. This reprogramming and changeover take time to accomplish, and there is a period of nonproductive time followed by a production run for each new batch. Production rates in programmable automation are generally lower than in fixed automation, because the equipment is designed to facilitate product changeover rather than for product specialization. A numerical-control machine tool is a good example of programmable automation. The program is coded in computer memory for each different product style, and the machine tool is controlled by the computer program.Industrial robots are another example.

Flexible automation is an extension of programmable automation. The disadvantage with programmable automation is the time required to reprogram and change over the production equipment for each batch of new product. This is lost production time, which is expensive. In flexible automation, the variety of products is sufficiently limited so that the changeover of the equipment can be done very quickly and automatically. The reprogramming of the equipment in flexible automation is done off-line; that is, the programming is accomplished at a computer terminal without using the production equipment itself. Accordingly, there is no need to group identical products into batches; instead, a mixture of different products can be produced one right after another.

- Match the following words and word combinations with their definitions:

| - Facility | - capable of bending easily without breaking |
| --- | --- |
| - flexible | - not producing or able to produce goods, crops, or economic benefit |
| - initial | - a place, amenity, or piece of equipment provided for a particular purpose |
| - disadvantage | - a quantity or consignment of goods produced at one time |
| - off-line | - an unfavorable circumstance or condition that reduces the chances of success or effectiveness |
| - batch | - not controlled by or directly connected to a computer or the Internet |
| - nonproductive | - existing or occurring at the beginning |

- Say whether the following statements are true or false:

- Three types of automation in production can be distinguished.

- Programmable automation is a form of automation for producing products in batches.

- The disadvantage with programmable automation is their being too complicated.

- Match heads and tales:

| - The products are made … | - …for producing products in batches |
| --- | --- |
| - Flexible automation is... | - … a good example of programmable automation. |
| - Examples of fixed automation include … | - …in batch quantities ranging from several dozen to several thousand units at a time. |
| - Programmable automation is a form of automation … | - …so that the changeover of the equipment can be done very quickly and automatically. |
| - In flexible automation, the variety of products is sufficiently limited… | - machining transfer lines found in the automotive industry, automatic assembly machines, and certain chemical processes. |
| - A numerical-control machine tool is… | - …an extension of programmable automation. |

- Answer the questions:

- What is programmable automation?

- What is the disadvantage of programmable automation?

- What is the difference between flexible and programmable automation?

- Make a short summary of the text.

###### 6 Automated production lines

An automated production line consists of a series of workstations connected by a transfer system to move parts between the stations. This is an example of fixed automation, since these lines are typically set up for long production runs, perhaps making millions of product units and running for several years between changeovers. Each station is designed to perform a specific processing operation, so that the part or product is constructed stepwise as it progresses along the line. A raw work part enters at one end of the line, proceeds through each workstation, and emerges at the other end as a completed product. In the normal operation of the line, there is a work part being processed at each station, so that many parts are being processed simultaneously and a finished part is produced with each cycle of the line. The various operations, part transfers, and other activities taking place on an automated transfer line must all be sequenced and coordinated properly for the line to operate efficiently. Modern automated lines are controlled by programmable logic controllers, which are special computers that facilitate connections with industrial equipment (such as automated production lines) and can perform the kinds of timing and sequencing functions required to operate such equipment.

Automated production lines are utilized in many industries, most notably automotive, where they are used for processes such as machining and pressworking. Machining is a manufacturing process in which metal is removed by a cutting or shaping tool, so that the remaining work part is the desired shape. Machinery and motor components are usually made by this process. In many cases, multiple operations are required to completely shape the part. If the part is mass-produced, an automated transfer line is often the most economical method of production. The many separate operations are divided among the workstations. Transfer lines date back to about 1924.

Pressworking operations involve the cutting and forming of parts from sheet metal. Examples of such parts include automobile body panels, outer shells of major appliances (e.g., laundry machines and ranges), and metal furniture (e.g., desks and file cabinets). More than one processing step is often required to complete a complicated part. Several presses are connected together in sequence by handling mechanisms that transfer the partially completed parts from one press to the next, thus creating an automated pressworking line.

- Match the following words and word combinations with their definitions:

| - changeover | - make (an action or process) easy or easier |
| --- | --- |
| - stepwise | - a change from one system or situation to another |
| - simultaneously | - the necessary items for a particular purpose, or the process of supplying someone or something with necessary equipment |
| - Facilitate | - an external capsule of a vehicle usually made of thin piece of metal |
| - equipment | - in a series of distinct stages; not continuously |
| - shaping tool | - a tool that gives a desired form to metal by hammering or pressure |
| - outer shells | - at the same time |

- Say whether the following statements are true or false:

- An automated production line consists of a series of workstations connected by a transfer system to move parts between the stations.

- One processing step is often required to complete a complicated part.

- Machining is a manufacturing process in which metal is removed by a cutting or shaping tool, so that the remaining work part is the desired shape.

- Match heads and tales:

| - Each station is designed to perform a specific processing operation… | - …are divided among the workstations. |
| --- | --- |
| - The various operations, part transfers, and other activities taking place on an automated transfer line... | - …so that the part or product is constructed stepwise as it progresses along the line. |
| - In many cases, multiple operations… | - …is often required to complete a complicated part. |
| - More than one processing step… | - ...must all be sequenced and coordinated properly for the line to operate efficiently. |
| - The many separate operations… | - …. and emerges at the other end as a completed product. |
| - A raw work part enters at one end of the line, proceeds through each workstation, | - …are required to completely shape the part. |

- Answer the questions:

- What is automated production line?

- What kind of process is machining?

- What do the pressworking operations involve?

- Make a short presentation on the information of the text.

###### 7 Automated assemblies

Assembly operations have traditionally been performed manually, either at single assembly workstations or on assembly lines with multiple stations. Owing to the high labor content and high cost of manual labor, greater attention has been given in recent years to the use of automation for assembly work. Assembly operations can be automated using production line principles if the quantities are large, the product is small, and the design is simple (e.g., mechanical pencils, pens, and cigarette lighters). For products that do not satisfy these conditions, manual assembly is generally required.

Automated assembly machines have been developed that operate in a manner similar to machining transfer lines, with the difference being that assembly operations, instead of machining, are performed at the workstations. A typical assembly machine consists of several stations, each equipped with a supply of components and a mechanism for delivering the components into position for assembly. A workhead at each station performs the actual attachment of the component. Typical workheads include automatic screwdrivers, staking or riveting machines, welding heads, and other joining devices. A new component is added to the partially completed product at each workstation, thus building up the product gradually as it proceeds through the line. Assembly machines of this type are considered to be examples of fixed automation, because they are generally configured for a particular product made in high volume.

- Match the following words and word combinations with their definitions:

| - assembly | - the amount or number of a material or abstract thing not usually estimated by spatial measurement |
| --- | --- |
| - manual | - an extra part or extension that is or may be attached to something to perform a particular function |
| - quantity | - place of or as an alternative to |
| - multiple | - the action of fitting together the component parts of a machine or other object |
| - attachment | - having or involving several parts, elements, or members |
| - transfer | - relating to or done with the hands, operated or controlled by hand, rather than automatically or electronically |
| - instead of | - move from one place to another |

- Say whether the following statements are true or false:

- Assembly operations have traditionally been performed automatically.

- For making such products as mechanical pencils, pens, and cigarette lighters usually automated assembly operations are used.

- The difference of automated assembly machines from machining transfer lines is that the first are performed at the workstations.

- A typical assembly machine consists of only one station.

- Match heads and tales:

| - Assembly operations… | - …and other joining devices. |
| --- | --- |
| - For products that do not satisfy these conditions, | - …performs the actual attachment of the component. |
| - Assembly operations can be automated using production line principles… | - …the partially completed product at each workstation |
| - A workhead at each station… | - ...manual assembly is generally required. |
| - Typical workheads include automatic screwdrivers, staking or riveting machines, welding heads… | - …have traditionally been performed manually. |
| - A new component is added to… | - …if the quantities are large, the product is small, and the design is simple |

- Answer the questions:

- What are the main principles of automated assembly?

- What is the difference between automated assembly machines and machining transfer lines?

- What do the typical workheads include?

- Make a short summary of the text.