Lecture 12

Application of risk theory to assess the level of security. Methodology of analysis and risk assessment. Criteria of acceptable risk

"Risk" is now increasingly being used to assess the impact of negative factors of production. This is due to the fact that the risk of a quantitative characteristic of the implementation of the dangers can be used to assess the state of working conditions, the economic damage as defined accidents and illnesses at work, forming a system of social policy at work (providing compensation benefits).

For risk characteristic of surprise, sudden onset of a dangerous situation that offers quick decisive actions to eliminate or reduce the impact of hazards.

The risk - quantitative characteristic of dangerous activity generated by a specific human activity, ie, the number of deaths, the number of cases, the number of temporary and permanent disability (disability) caused by the action of a person specific hazard (electric current, harmful substance, move objects, criminal elements of society and OE), assigned a certain number of citizens (employees) for a specific period of time. The value of the risk of a particular hazard can be obtained from the statistics of accidents, cases, cases of violence against members of the society for different periods of time: shift, day, week, quarter, year.

Preliminary hazard analysis (PHA), which consists in identifying hazards, determining the system or events that can cause a hazardous condition, the characterization of hazards in accordance with consequences caused by them.

Preliminary hazard analysis is carried out in the following order:
- study of technical characteristics of an object, system, process, used energy sources, working environments, materials and install them damaging properties;
- provide the regulatory-technical documentation which applies to this technical object, system, process;

- inspect the existing technical documentation for compliance with the rules and regulations of security;

- make a list of hazards, which indicate the identified hazards, damaging factors, potential accident, identified shortcomings.

Overall, PHA is a first attempt to identify equipment and technical systems (in its initial version) and some of the events that can cause hazards. This analysis is at an early stage of development of the system. A detailed analysis of possible events is typically performed using fault tree, once the system is fully defined.

Analysis of the consequences of failure (ACF) is a qualitative method of hazard identification based on systems approach and having the character of the forecast. ACF is the analysis of the inductive type, which systematically, through successive consideration of one element over the others, analyzed all possible types of failures or emergency situations and identifies their resulting effect on the system.

Separate emergency and failure modes of elements, to determine their impact on other nearby elements and the system as a whole. ACF performed in the following order:

- technical system (object) is divided into components;
- for each component, identify possible failures;
- exploring the potential of accidents which may cause cracks on the investigated object;
- failures rank the hazards and develop preventive measures.

This method can be used to evaluate dangerous potential of any technical object. The test results show that failures can be collected data on the frequency of failures, are necessary to quantify the level of danger of the object.
A hazard analysis using the "fault tree" of potential accident (HAFT) allows to identify combinations of failures (faults) of equipment, human error and external (anthropogenic, natural) influences, leading to the main event (emergency situation). ADP performed in the following order:
- choose a potential event-an accident or failure that may cause accidents;
- identify all the factors that can lead to a given accident, including all potential incidents;
- the results of this analysis build a directed graph-tree node (root) which contains a potential accident.

The hazard analysis method of potential differences (HAMPD) includes the procedure of artificially creating deviations using key words. To do this, break the technological process or technical system into its constituent parts and creating with the help of key words deviation, systematically studying their potential causes and the consequences to which they lead in practice.

Analysis of human error (AHE) is one of the most important elements of the methodology assessment of risks taking into account the human factor, to characterize how errors, initiating or aggravating an emergency situation and the ability of personnel to perform corrective actions on accident management.

Risk assessment - an analysis of the origin (origin) and the extent of risk in a particular situation.

The probability of an unwanted event, taking into account the size of possible damage caused by the event.

To distinguish:

• individual risk - the probability of death of a person in this type of activity;

• social risk - the dependence of the number of people who died on the frequency of occurrence of the event causing the loss of these people.

Risk analysis and risk analysis - the systematic use of available information to identify hazards and assess risks to individuals or groups of people, property or the environment.

The concept of risk always consists of two elements: the frequency with which the hazardous event and the consequences of the hazardous event. A risk analysis, in turn, is to identify (identification) of hazards and risk assessment when a threat is understood as a source of potential injury or harm or a situation with the possibility of damage, and a hazard identification - the process of identifying and recognizing that the danger exists, and determination of its characteristics. Application of the concept of risk, therefore, allows you to translate into the category of risk of the measured categories. The risk of danger has actually measure.

Risk assessment includes a frequency analysis, impact analysis, and combinations thereof.

Human

Instant impact

Gradual exposure

Death

Acute poisoning

Injury

Disease

Reducing health

Fatigue

Biological

Сhemical

Рhysical

Psychophysiological

Сhemical

Hydraulic

energy

Thermal

Mechanical

Harmful

Dangerous

Unwanted sequela

Factors causing the occurrence of hazards

Dangerous area

Danger

Causes

Dangerous situations

Fig 1. Decomposition dangerous situation

The next stage of risk analysis - the identification of hazards. The main task - identifying (on the basis of information on this subject, examination results and experience of similar systems), and a clear description of all the dangers inherent in the system. It also carried out a preliminary assessment of the hazards in order to select further areas of activity:

- Discontinue further analysis in view of the insignificance of the dangers;

- To carry out a more detailed risk analysis;

- To make recommendations to reduce risks.

In principle, the risk analysis process can be terminated at the stage of identification of hazards.

It should be emphasized that the complex and expensive calculations often yield risk value, which is very low accuracy. The greatest amount of safety recommendations is produced using high-quality (engineering) methods of risk analysis, allowing to achieve the main objectives of the risk-analysis using a smaller amount of information and labor costs. However, quantitative risk assessment methods are always very helpful, and in some cases - and only allowed, in particular, to compare the hazards of different nature or especially dangerous examination of complex technical systems.

Development of recommendations for risk reduction (risk management) is the final step in the risk analysis. Recommendations can recognize the existing risk is acceptable or specify measures to reduce the risk, ie. E. According to his management measures. risk management measures can be technical, operational or organizational nature.