Landslides are classified by their type of movement. The four main types of movement are:

- falls (обвал)

- topples(обвал с переворачиванием породы )

- slides (скольжение)– rotational and translational

- flows (оплывины)

Landslides (оползни)

Landslides can be classified as just one of these movements or, more commonly, can be a mixture of several. Geologists also refer to the type of material involved in the movement e.g. rock, debris, earth.

## Falls

Falls are landslides that involve the collapse of material from a cliff or steep slope. Falls usually involve a mixture of free fall through the air, bouncing or rolling. A fall type landslide results in the collection of rock or debris near the base of a slope.

A good example of a fall landslide is the

Falls: the rock mass descends mostly through air by free fall, bouncing or rolling, after being separated from the rest of the slope.

## Topples

Topple failures involve the forward rotation and movement of a mass of rock, earth or debris out of a slope. This kind of slope failure generally occurs around an axis (or point) at or near the base of the block of rock.

.

Topples: movements of rock, debris or earth masses by forward rotation about a pivot point. BGS ©UKRI. All rights reserve

A topple often results in the formation of debris or a debris cone at the base of the slope; this pile is called a talus cone. New talus cones don’t have any plants growing on them. Old talus cone can have weeds and even trees on them.

Slides

Slides are characterised by a failure of material at depth and then movement by sliding along a rupture or slip surface. There are two types of slide failure, rotational slides (slumps) and translational (planar) slides:

#### Rotational slides

A good example of a rotational landslide is the Holbeck Hall landslide, in Scarborough North Yorkshire, England. The 1 million tonnes of cliff failed as a rotational landslide over a couple of days in June 1993 and destroyed the Hotel at the top of the cliff. The rotated blocks can be seen as grass covered ‘benches’ in the photograph. BGS ©UKRI. All rights reserved.

A slide type landslide is a down-slope movement of material that occurs along a distinctive rupture or slip surface. If this slip surface is listric (curved or spoon-shaped) the slide is said to be rotational. The slip surface tends to be deeper than that of other landslide types and not structurally controlled. These landslides are characterized by a prominent main scarp and back-tilted bench or block at the top with limited internal deformation. Below this, movement is more or less rotational about an axis.

#### Translational slides

A translational or planar landslide is a down-slope movement of material that occurs along a distinctive planar surface of weakness such as a fault, joint or bedding plane.    Some of the largest and most damaging landslides on earth are translational.  These landslides occur at all scales and are not self-stabilising.  They can be very rapid where discontinuities are steep.

## Flows

Flows are landslides that involve the movement of material down a slope in the form of a fluid. Flows often leave behind a distinctive upside-down funnel shaped deposit where the landslide material has stopped moving.  There are different types of flows: mud, debris and rock (avalanches).  Two of the most common in the UK are mud flows and debris flows.  Mud flows can be found on the south coast of England, often associated with larger complex landslides.  Debris flows can be very rapid and usually occur on steep slopes.

### Why do landslides happen?

A landslide may occur because the strength of the material is weakened. This reduces the power of the ‘glue’ that cements the rock or soil grains together. Located on a slope, the rock is then no longer strong enough to resist the forces of gravity acting upon it.

What can increase the chance of a landslide?

Several factors can increase a slope’s susceptibility to a landslide event:

- water — adding water to the material on a slope, makes a landslide more likely to happen. This is because water adds weight, lowers the strength of the material and reduces friction, making it easier for material to move downslope

- erosion processes — such as coastal erosion and river erosion — if the bottom of a slope is continually eroded by the sea or a river, the slope will eventually become too steep to hold itself up

- steepness of slope — the slope angle is a key factor as far as landslides are concerned.  Any change to this that makes it steeper (such as coastal erosion) increases the likelihood of a landslide

- type of ‘rocks’ — soft rock such as mudstone or hard rock such as limestone — the type of rocks in the slope, and their combination

- shape of the rock ‘grains’

- jointing and orientation of bedding planes

- arrangement of the rock layers

- weathering processes — for example freeze-thaw reduces the stickiness (cohesion) between the rock grains

- lack of vegetation which would help bind material together

- flooding

- volcanoes and earthquake activity nearby

- human activity — mining, traffic vibrations or urbanisation which changes surface water drainage patterns

# Landslides

Landslides can be caused by earthquakes or volcanoes, but in  Queensland,  they’re generally caused by heavy rain. The rain saturates the soil on a  hillside—often where there has been human activity (e.g. construction where  trees and plants have been removed)—past the point where any remaining vegetation  can support the soil’s weight against the force of gravity. The top saturated  layer of soil then slips down the hill—taking whatever is on the land with it.

Landslides may be exceptionally slow—moving only centimetres  a year—or fast, with a sudden and total collapse, moving millions of tonnes of  debris.

The  distance a landslide travels also varies greatly—from a few centimetres in  'ground slumps', to many kilometres when large mud flows follow river valleys.

## Preparing for landslides

You can protect yourself, your family and your property by  doing the following:

- Before occupying a building, check with the local council and neighbours for the area's history of landslides or instability.

- In steep areas, look for the tell-tale signs of  ground movement, such as trees tilting (down-slope), water seepage and breaks  in the ground.

- If outdoors always heed warning signs, and avoid the tops and bases of cliffs and  embankments, especially where there are signs of loose rocks or debris. Never  stand or sit on rock overhangs unless you are sure they can bear your weight.

## During a landslide

- If indoors—shelter  at the least-affected end of the building under a strong table or bench (if  possible, use a mattress for extra protection). Hold on firmly and stay put  until all land movement has stopped.

- If outdoors and a landslide threatens, move quickly from its path and keep clear of banks,  trees, powerlines and poles.

A landslide is the movement of rock, earth, or debris down a sloped section of land. Landslides are caused by rain, earthquakes, volcanoes, or other factors that make the slope unstable.

Geologists, scientists who study the physical formations of the Earth, sometimes describe landslides as one type of mass wasting. A mass wasting is any downward movement in which the Earth's surface is worn away. Other types of mass wasting include rockfalls and the flow of shore deposits called alluvium.

Near populated areas, landslides present major hazards to people and property. Landslides cause an estimated 25 to 50 deaths and $3.5 billion in damage each year in the United States.

What Causes Landslides?

Landslides have three major causes: geology, morphology, and human activity.

Geology refers to characteristics of the material itself. The earth or rock might be weak or fractured, or different layers may have different strengths and stiffness.

Morphology refers to the structure of the land. For example, slopes that lose their vegetation to fire or drought are more vulnerable to landslides. Vegetation holds soil in place, and without the root systems of trees, bushes, and other plants, the land is more likely to slide away.

A classic morphological cause of landslides is erosion, or weakening of earth due to water. In April 1983, the town of Thistle, Utah, experienced a devastating landslide brought on by heavy rains and rapidly melting snow. A mass of earth eventually totaling 305 meters wide, 61 meters thick, and 1.6 kilometers long (1,000 feet wide, 200 feet thick, and one mile long) slid across the nearby Spanish Fork River, damming it and severing railroad and highway lines. The landslide was the costliest in U.S. history, causing over $400 million in damage and destroying Thistle, which remains an evacuated ghost town today.

Human activity, such as agriculture and construction, can increase the risk of a landslide. Irrigation, deforestation, excavation, and water leakage are some of the common activities that can help destabilize, or weaken, a slope.

Types of Landslides

There are many ways to describe a landslide. The nature of a landslide's movement and the type of material involved are two of the most common.

Landslide Movement

There are several ways of describing how a landslide moves. These include falls, topples, translational slides, lateral spreads, and flows.

In falls and topples, heavy blocks of material fall after separating from a very steep slope or cliff. Boulders tumbling down a slope would be a fall or topple.

In translational slides, surface material is separated from the more stable underlying layer of a slope. An earthquake may shake the loosen top layer of soil from the harder earth beneath in this type of landslide.

A lateral spread or flow is the movement of material sideways, or laterally. This happens when a powerful force, such as an earthquake, makes the ground move quickly, like a liquid.

Landslide Material

A landslide can involve rock, soil, vegetation, water, or some combination of all these. A landslide caused by a volcano can also contain hot volcanic ash and lava from the eruption. A landslide high in the mountains may have snow and snowmelt.

Volcanic landslides, also called lahars, are among the most devastating type of landslides. The largest landslide in recorded history took place after the 1980 eruption of Mount St. Helens in the U.S. state of Washington. The resulting flow of ash, rock, soil, vegetation and water, with a volume of about 2.9 cubic kilometers (0.7 cubic miles), covered an area of 62 square kilometers (24 square miles).

Other Factors

Another factor that might be important for describing landslides is the speed of the movement. Some landslides move at many meters per second, while others creep along at an centimeter or two a year. The amount of water, ice, or air in the earth should also be considered. Some landslides include toxic gases from deep in the Earth expelled by volcanoes. Some landslides, called mudslides, contain a high amount of water and move very quickly. Complex landslides consist of a combination of different material or movement types.

landslide

A landslide can topple trees, rocks, even buildings.

Photograph by Danielle Stevens, My Shot