5. Wave Energy Buoy

DEFINITION: device / wave energy – electricity

MAIN COMPONENTS: copper coil, magnetic shaft, fiberglass container, cable

LOCATION: copper coil in buoy floats on sea surface; magnetic shaft attached by cable to seabed

MAIN SPECIFICATIONS: buoy 4m (H); 35 m above seabed; 35 m between each buoy; 1.5-3 km from shore; 100 kw power

OPERATION: wave oscillates --- magnetic shaft/ reciprocate (explain to listeners: this means “move up and down”) --- shaft/ move/ through copper coil --- induce/ electric current

BENEFITS: wave energy v wind energy? More reliable; renewable; no fossil fuels

PROBLEM: about 500 buoys needed; could damage ships, fish

VOCABULARY

copper I ['kɔpə] coil I [kɔɪl]       медь спираль

magnetic shaft                      магнитная ось

cable ['keɪbl]              	  канат, трос

buoy [bɔɪ]                    	буй, бакен

induce [ɪn'djuːs]	 вызывать; стимулировать; приводить (к чему-л.)

reliable [rɪ'laɪəbl]	 надёжный; верный, испытанный

fossil ['fɔs(ə)l] fuel [fjuːəl] 	ископаемое топливо

damage ['dæmɪʤ] 	вред, повреждение; дефект, поломка; убыток, ущерб, урон

6. WAVE ENERGY STATION

DEFINITION: device/convert energy from waves --- electrical power

MAIN COMPONENTS: air chamber, turbine, generator, valves

LOCATION: fixed to the seabed; close to shore

MAIN SPECIFICATIONS: 35 m length x 35 m width; 485 tonnes weight; made of steel; 200 m from shore

OPERATION: wave enters chamber --- water rises and falls in chamber --- water forces air in and out of hole --- moving air makes turbine rotate --- turbine turns generator

BENEFITS: free energy; no fuel needed; no waste produced; not expensive to operate

PROBLEM: noisy; must be able to resist rough weather

VOCABULARY

valve [vælv]				 клапан; вентиль; золотник; затвор; створка

expensive [ɪk'spen(t)sɪv ]		дорогой, дорогостоящий

turbine 				['tɜːbaɪn] ; турбина

rough weather 			непогода, буря, ненастье

7. WAVE ROLLER

DEFINITION: device/change/wave energy --- electricity

MAIN COMPONENTS: base plate, moving plate, piston pump, hydraulic pipes, turbine, generator

LOCATION: base plate fixed to the seabed; hydraulic pipes on seabed; turbine and generator on shore

MAIN SPECIFICATIONS: moving plate approximately 1 m x 1 m; depth 7-15m below sea surface; each plate produces 13 kw power

OPERATION: waves move forward and backward along seabed --- plate oscillates --- pistons move in reciprocating motion --- turbine rotates --- generator produces electricity

BENEFITS: Wave Roller invisible because on seabed; no noise; does not interfere with ships

VOCABULARY

base plate 						опорная плита

piston pump 					поршневой насос

turbine ['tɜːbaɪn] 					турбина

hydraulic [haɪ'drɔːlɪk]                                    Гидравлический

approximately [ə'prɔksɪmətlɪ]                        приблизительно

8. wave energy converter