0:07

hello I'm John marks dowdy president and

00:11

CEO of riverside resources this is

00:13

another episode of John marks geo

00:16

tutorial today I'm here with Mickey

00:18

fault the mercenary geologist Mickey's

00:21

here to help us go through the steps for

00:23

making a geologic map covering different

00:26

steps of prospecting and coming up with

00:28

a drill target Mickey

00:30

thank you for joining me thanks for

00:32

inviting me john mark looking forward to

00:34

going through the steps today look at me

00:35

too now we're gonna go through the

00:38

various steps in making a geologic map

00:40

but the first thing we need to do is get

00:42

the tools Mickey what are the tools we

00:44

need to make a geologic map well there

00:46

are several tools that we use John Mark

00:48

and we start with our map board and as

00:51

you see here I have a simple masonite

00:53

board of upon which I've mounted a

00:56

topographic map and over that

00:58

topographic map I have a mylar overlay

01:01

and mylar is thin plastic upon which we

01:05

plot our geology with a set of pencils

01:08

including regular LED pencils and

01:11

colored pencils we locate ourselves with

01:15

the GPS which gives us our exact

01:17

position on the topo map and we use the

01:22

brunt and compass Illustrated here which

01:26

is a special geologists compass which

01:29

allows us to measure both the attitude

01:33

and the trend of the rocks the trend and

01:36

the attitude in which the rocks are

01:38

sitting also of importance we're working

01:41

as you can see in the desert is the

01:44

water and there's nothing that is more

01:47

important to a geologist working in the

01:49

in the desert especially in the

01:51

summertime than to stay hydrated so he

01:54

is at his peak efficiency in making the

01:56

geological map well let's go make a map

01:59

Mickey let's do that John mark here we

02:03

are at a geologic contact with Mickey

02:05

what do we do when we're doing a

02:07

geologic map when we get to the contact

02:09

how do you document it well the first

02:11

thing we have to do is figure out what

02:14

kind of rock types we're dealing at so

02:16

we have right here two kinds of rocks

02:18

unless

02:20

we have this white rock here and so the

02:24

first thing we're going to do is we're

02:26

going to take out our hand lens and

02:28

we're going to identify this rock type

02:30

and we see this is white bull quartz one

02:33

of our Rock units in the other map kind

02:37

of rock we have is this kind of light

02:41

brownish tan rock and we look at that

02:43

and we say that's a quartz aracite

02:46

schist

02:46

so now we know what kind of rocks we

02:49

have here and the next step is to

02:51

determine where we are on the topo map

02:54

so we take out our GPS we punch in the

02:59

you take the coordinates so with the GPS

03:03

then so if you take the coordinates then

03:05

you're going to put them on your map

03:06

aren't you that's right so we now know

03:09

where we are on the topo map take out

03:12

our pencil and we draw the contact on

03:17

the geology map very important concept

03:21

make the map in the field plot your data

03:24

in the field so if you're going to plot

03:26

this data in the field let's go look at

03:29

one of the D outcrops and see some of

03:30

the observations that you'd make on the

03:32

outcrop okay let's do that John Mark

03:34

here we are sitting on an outcrop with

03:37

Mickey fault he's going to go through

03:39

how to take an attitude and how what are

03:41

some of the observations you're making

03:43

on the outcrop once again the first

03:45

thing we need to determine John mark is

03:47

the rock type we're dealing with take my

03:50

rock hammer knock up a bit of rock

03:54

observe the minerals in the rock

03:57

identified the rock again a quartz

04:00

aracite shifts but this time with very

04:02

strong iron oxides which tends to

04:06

indicate that there is mineralization in

04:09

this rock the next thing I need to do

04:11

John Mark is I need to to determine the

04:14

trend and attitude of the rock and I do

04:16

that with my geologist compass a Brunton

04:19

compass and the first thing I do is I

04:22

take the strike of the rock and I do

04:24

that you notice I have my field book

04:26

laying here to get a very flat surface

04:28

and I measure the trend and the trend is

04:32

basically

04:33

West here an east-west strike in

04:36

geological parlance and then I need to

04:39

take the attitude of the rock and that's

04:41

the way the rocks are are dipping into

04:44

the ground so I hope that the viewers

04:47

can see that this rock dips very

04:50

shallowly toward the north and I take my

04:52

brunt and compass and I lay it on the

04:54

rock and I record this angle and this is

04:57

very important in allowing us to

05:01

determine the direction and the angle

05:03

that we need to drill our holes on our

05:05

target so making you take that

05:07

information the glad it's the mad

05:10

attitude the strike and dip you're gonna

05:13

put it on your map you've drawn the

05:15

geologic contacts that's gonna help you

05:17

to tell some of the folds and faults to

05:19

make a more complete geologic map Here I

05:26

am with Mickey fault mercenary geologist

05:29

on an outcrop that has alteration the

05:31

red is hematite the white is a quartz

05:34

vein we want to take a sample here to

05:36

find out if it has gold Mickey how do we

05:38

do that well the first thing we want to

05:41

do is we want to mark the location of

05:43

our sample or that we're going to take

05:46

on our GPS and we're using our GPS and

05:49

are in our geology map so we do that

05:52

then because I'm a geologic map or I

05:55

need to focus on geologic mapping and so

05:58

we we take a piece of orange flagging

06:00

tape and we wrap it around a rock we've

06:04

already located it on our map on a

06:06

separate overlay which contains all of

06:09

our samples and we're going to bring our

06:11

geotechnical crew in to sample we know

06:14

that with all these iron oxides here we

06:17

have a quartz vein next to us that's

06:18

been prospected and that this possibly

06:21

contains gold so we we will develop our

06:25

drill targets in addition to the mapping

06:27

with the geochemical sampling well

06:30

Mickey now we've seen the different

06:32

parts to make a geologic map and we

06:35

bring those together to do the

06:36

interpretation in a cross-section what

06:38

are the key components

06:39

you think about in making a

06:40

cross-section well we've determined

06:42

through our observational data we've

06:44

determined the boundaries between rock

06:46

types we've determined our favorable

06:49

rock type for hosting gold

06:51

mineralization we've determined the

06:53

attitude and trend of those rock types

06:55

now

06:56

we practice the art of geology and

06:59

that's interpreting the rock we need to

07:01

separate our observational data from our

07:04

interpretation and we do that by

07:06

projecting a two-dimensional surface in

07:09

the third dimension with a cross section

07:11

which tells us the at the angle that we

07:15

need to drill the direction we need to

07:17

drill and the depth we need to drill to

07:20

reach our gold target well here at

07:22

Sugarloaf peak we've done the drilling

07:24

we've to used the cross sections and now

07:27

in this video we've gone through the

07:29

various steps to make a geologic map

07:31

leading to drilling and doing the

07:33

interpretations

Английский (создано автоматически)