Subject 1. The Calculation of the failure rate

Often an assessment of a random variable can be obtained using the binomial distribution.

Suppose that the set of serviceable components include the proportion of q and the proportion defective p, and

q  + p= 1						            (1)

Furthermore, if the sample contains two elements of a set comprising a serviceable and q% p% defective parts, for such combinations of a number of samples produced:

| Exodus | Probability | The probability of occurrence |  |
| --- | --- | --- | --- |
|  |  | Еach case | Combinations |
| 2 serviceable | Good Good | qq | q2 |
| 1 faulty | Good Bad Bad Good | qp pq | 2qp |
| 2 serviceable | Bad Bad | pp | p2 |

If you add up all the expressions of Box 4, we get the expression, which is a second-degree binomial

(2)

The degrees can be and more. So when the number of degrees necessary to obtain the product of 4

(3)

Obviously, the power of the parameter q varies from 0 to 4, and the degree of parameter p from 0 to 4, and the degree of binomial coefficients can be determined by the Pascal triangle:

| Sample size | Coefficients |
| --- | --- |
| 1 2 3 4 5 6 | 1 1 2 1 1 3 3 1 1 4 6 4 1 1 5 10 10 5 1 1 6 15 20 15 6 1 |

1.1 Assignment

Using the binomial distribution deter- probability of occurrence in samples 0, 1, 2, 3 or 4 of defective products, provided one hundred sample size N, was taken from a large consignment containing X% of defective products. Calculation to produce the embodiment of the table. Edition number is determined by the last digit of a student record book.

Table 1 - Options jobs

| Option | The percentage of defective products, X | The amount of N samples |
| --- | --- | --- |
| 1 | 2 | 4 |
| 2 | 3 | 5 |
| 3 | 4 | 4 |
| 4 | 5 | 5 |
| 5 | 6 | 4 |
| 6 | 7 | 5 |
| 7 | 8 | 4 |
| 8 | 9 | 5 |
| 9 | 11 | 4 |
| 10 | 12 | 5 |

1.2 Example solutions

Initial data. Suppose that a sample of size N = 4 was taken from a large batch, containing 10% of defective products. What is the probability of occurrence in samples 0, 1, 2, 3 or 4 faulty product

Decision.

The probability of occurrence of defective products q4=(09)4=0,6561

1 defective product 4q3p=4(09)3 (0.1)=0,2916

2-defective products 6q2p2=4 6 (09)2 (0.1)2=0.0486

3- defective products 4qp3=4(09) (0.1)3=0.0036

4- defective products p4=4(0.1)4=0.0001

The total probability = 1