PHYSICAL HAZARDS

Read and translate the text

Physical hazards include excessive levels of ionizing and non-ionizing electromagnetic radiation, noise, vibration, illumination, and temperature.

In occupations where there is exposure to ionizing radiation, time, distance, and shielding are important tools in ensuring worker safety. Danger from radiation increases with the amount of time one is exposed to it; hence, the shorter the time of exposure the smaller the radiation danger.

Distance also is a valuable tool in controlling exposure to both ionizing and non-ionizing radiation. Radiation levels from some sources can be estimated by comparing the squares of the distances between the worker and the source. Shielding also is a way to protect against radiation. The greater the protective mass between a radioactive source and the worker, the lower the radiation exposure.

Non-ionizing radiation also is dealt with by shielding workers from the source. Sometimes limiting exposure times to non-ionizing radiation or increasing the distance is not effective. Laser radiation, for example, cannot be controlled effectively by imposing time limits. An exposure can be hazardous that is faster than the blinking of an eye. Increasing the distance from a laser source may require miles before the energy level reaches a point where the exposure would not be harmful.

Noise, another significant physical hazard, can be controlled by various measures. Noise can be reduced by installing equipment and systems that have been engineered, designed, and built to operate quietly. Substituting quiet work methods for noisy ones is another significant way to reduce noise. Also, treating floors, ceilings, and walls with acoustical material can reduce reflected or reverberant noise. In addition, erecting sound barriers at adjacent work stations around noisy operations will reduce worker exposure to noise generated at adjacent work stations.

It is also possible to reduce noise exposure by increasing the distance between the source and the receiver, by isolating workers in acoustical booths, limiting workers' exposure time to noise, and by providing  hearing  protection.

1) Mark these statements T or F according to the information in the text.

1. Danger from radiation decreases with the amount of time one is exposed to it.

2. Laser radiation cannot be controlled effectively by imposing time limits.

3. Substituting quiet work methods for noisy ones is significant way to eliminate noise.

4. Shielding is a way to protect against temperature.

5. Distance also is a valuable tool in controlling exposure to both ionizing and non-ionizing radiation.

2) Match heads and tails

| 1. Non-ionizing radiation also is dealt with | A. the lower the radiation exposure. |
| --- | --- |
| 2. In occupations where there is exposure to ionizing radiation, time, distance, and shielding | B. can reduce reflected or reverberant noise. |
| 3. Treating floors, ceilings, and walls with acoustical material | C. the smaller the radiation danger. |
| 4. The greater the protective mass between a radioactive source and the worker, | D. by shielding workers from the source. |
| 5. The shorter the time of exposure | E. are important tools in ensuring worker safety. |

3) Match these terms with their definitions

| 1. radiation | A. a thing likely to cause injury, etc |
| --- | --- |
| 2. hazard | B. to place (machinery, equipment, etc.) in position and connect and adjust for use |
| 3. safety | C. the act providing somebody’s security |
| 4. install | D. the emission or transfer of radiant energy as particles, electromagnetic waves, sound, etc |
| 5. protection | E. freedom from danger or risk of injury |

4) Fill in the gaps

Noise, another significant physical (1) _______, can be controlled by various measures. Noise can be reduced by installing (2) _________ and systems that have been engineered, designed, and built to operate quetly. Substituting quiet work methods for noisy ones is another significant way to (3) ________ noise. Also, treating floors, ceilings, and walls with (4) _______ material can reduce reflected or reverberant noise. In addition, erecting sound barriers at adjacent work stations around noisy operations will reduce worker (5) ________to noise generated at adjacent work stations.

A. reduce

B. acoustical

C. hazard

D. exposure

E. equipment