Fire fighting

Fire fighting is a highly skilled operation and most medium and large firms have properly trained teams who can contain the fire locally until the professional brigade arrives. The best way you can help is to learn the correct fire drill, both how to give the alarm and how to leave the building.

It requires only one person to panic and run in the wrong direction to cause a disaster.

In an emergency never lose your head and panic.

Smoke is the main cause of panic. It spreads quickly through a building, reducing visibility and increasing the risk of falls down stairways. It causes choking and even death by asphyxiation. Smoke is less dense near the floor: as a last resort crawl. To reduce the spread of smoke and fire, keep fire doors closed at all times but never locked. The plastic materials used in the finishes and furnishings of modern buildings give off highly toxic fumes. Therefore it is best to leave the building as quickly as possible and leave the fire fighting to the professionals who have breathing apparatus.

Saving human life is more important than saving property.

If you do have to fight a fire there are some basic rules to remember. A fire is the rapid oxidation (burning) of flammable materials at relatively high temperatures. Figure 1 shows that removing the air (oxygen), or the flammable materials (fuel), or lowering the temperature will result in the fire ceasing to burn. It will go out. It can also be seen from Fig. 1 that different fires require to be dealt with in different ways.

| The 3 essentials to start a fire Note: Once the fire has started it produces sufficient heat to maintain its own combustion reactions and sufficient surplus heat to spread the fire | Remove heat When solids are on fire remove heat by applying water | Remove oxygen Liquids, such as petrol etc. on fire can be extinguished by removing oxygen with a foam or dry powder extinguisher | Remove fuel Electrical or gas fires can usually be extinguished by turning off the supply of energy |
| --- | --- | --- | --- |

Figure 1. How to remove each of the three items necessary to start a fire. (Note: Once the fire has started it produces sufficient heat to maintain its own combustion reaction and sufficient surplus heat to spread the fire)

Task 1. Finish the sentences:

- Before the professional brigade arrives …

- Smoke can cause…

- The spread of smoke and fire can be reduced, if you…

- The burning of plastic materials produces…

- It is more important to save human lives than …

Task 2. Mark the sentences as true/false

| Statement | T/F |
| --- | --- |
| - Burning of flammable materials at relatively high temperatures is called a fire. |  |
| - If you remove oxygen or fuel and lower the temperature, the fire will go out. |  |
| - You should deal with different fires in similar ways. |  |
| - When the fire starts it doesn’t have enough heat to maintain its own combustion reactions. |  |
| - The fire spreads because it has sufficient surplus heat. |  |
| - Applying water when solids are on fire removes oxygen. |  |
| - Using a foam or dry powder extinguisher on burning liquids, such as petrol will remove oxygen. |  |
| - You can stop electrical or gas fires if you turn off the supply of energy. |  |